RISC vs CISC

## ⚙️ 1. Basic Concept

| Feature | RISC (Reduced Instruction Set Computer) | CISC (Complex Instruction Set Computer) |
| --- | --- | --- |
| Philosophy | Use a small number of simple instructions that execute very fast. | Use a large number of complex instructions that can perform multiple operations. |
| Design Goal | Simplify the hardware and make execution faster. | Simplify the compiler and make each instruction more powerful. |
| Key Idea | “Do less, but do it faster.” | “Do more in one instruction.” |

## 🧩 2. Instruction Set

| Feature | RISC | CISC |
| --- | --- | --- |
| Number of Instructions | Small (around 30–100) | Large (hundreds or even thousands) |
| Instruction Complexity | Simple, fixed-format instructions | Complex, variable-length instructions |
| Instruction Size | Fixed (usually 32 bits) | Variable (8–64 bits or more) |
| Instruction Types | Few addressing modes and instruction formats | Many addressing modes and instruction formats |
| Examples of Operations | Load, store, add, subtract, branch | Memory-to-memory operations, string manipulation, etc. |

## 🧠 3. Execution and Performance

| Feature | RISC | CISC |
| --- | --- | --- |
| Clock Cycles per Instruction (CPI) | 1 (each instruction executes in one cycle, ideally) | Many (a single instruction may take multiple cycles) |
| Execution Speed | Faster (due to simpler instructions) | Slower (because of complex decoding) |
| Pipelining | Easy to implement because of uniform instruction length | Difficult due to variable instruction size |
| Microcode | Usually not used (direct hardware implementation) | Frequently used (instructions translated to micro-operations) |

## 🧮 4. Memory and Register Usage

| Feature | RISC | CISC |
| --- | --- | --- |
| Register Usage | Heavy use of registers (Load/Store architecture) | Many operations directly access memory |
| Memory Access | Only Load and Store instructions access memory | Many instructions can access memory (e.g., ADD A, [B]) |
| Instruction Fetches | More instructions fetched (each simpler) | Fewer instructions fetched (each does more work) |

## 🧰 5. Hardware and Design Complexity

| Feature | RISC | CISC |
| --- | --- | --- |
| Control Unit | Hardwired (simpler, faster) | Microprogrammed (complex, slower) |
| Decoder | Simple instruction decoding | Complex instruction decoding |
| Transistor Count | More used for registers | More used for microcode storage |
| Hardware Complexity | Simple | Complex |

## 💻 6. Compiler and Programming

| Feature | RISC | CISC |
| --- | --- | --- |
| Compiler Workload | More complex (compiler must break high-level instructions into multiple simple ones) | Easier (one instruction can represent a complex operation) |
| Instruction per Program | Larger (needs more instructions) | Smaller (each instruction does more work) |
| Code Density | Lower | Higher |
| Programming Ease (Assembly) | Slightly harder (more lines of code) | Easier (fewer lines of code) |

## 🔋 7. Examples of Processors

| RISC Examples | CISC Examples |
| --- | --- |
| ARM (used in smartphones) | Intel x86, AMD processors |
| MIPS | VAX |
| SPARC | Motorola 68000 |
| PowerPC | System/360 |

## 📊 8. Practical Example

Let’s say we want to compute

A = B + C

### In RISC:

LOAD R1, B     ; Load B into register R1

LOAD R2, C     ; Load C into register R2

ADD  R3, R1, R2; Add B and C → store in R3

STORE A, R3    ; Store result in A

👉 4 simple instructions.

### In CISC:

ADD A, B, C    ; Add B and C, store directly in A

👉 1 complex instruction.

## 🚀 9. Performance Summary

| Aspect | RISC | CISC |
| --- | --- | --- |
| Speed | High (simple operations, pipelined) | Moderate (complex decoding) |
| Power Efficiency | Better (simpler hardware) | Less efficient |
| Cost | Lower | Higher |
| Code Size | Larger | Smaller |

## 🧾 10. Modern Trend

Modern processors use a hybrid approach:

- Intel’s x86 (CISC) internally translates CISC instructions into RISC-like micro-operations (µops).

- ARM processors (RISC) now support some complex operations for efficiency.

So, in practice, the boundary between RISC and CISC has blurred — both borrow features from each other.

### 🧠 Summary Table (at a glance)

| Feature | RISC | CISC |
| --- | --- | --- |
| Instruction Set | Small | Large |
| Instruction Length | Fixed | Variable |
| Memory Access | Only Load/Store | Directly by instructions |
| Execution Time | One clock cycle | Multiple clock cycles |
| Pipelining | Easy | Difficult |
| Control Unit | Hardwired | Microprogrammed |
| Hardware Complexity | Simple | Complex |
| Example | ARM, MIPS | Intel x86, AMD |

### 📊 Difference Between Fixed-format and Variable-length Instructions

| বিষয় | Fixed-format Instruction | Variable-length Instruction |
| --- | --- | --- |
| অর্থ | প্রতিটি instruction-এর দৈর্ঘ্য একই থাকে (যেমন 32-bit)। | প্রতিটি instruction-এর দৈর্ঘ্য ভিন্ন হতে পারে (যেমন 1–15 byte)। |
| ব্যবহার হয় কোথায় | RISC আর্কিটেকচারে (যেমন ARM, MIPS, SPARC) | CISC আর্কিটেকচারে (যেমন Intel x86, AMD) |
| Instruction Decoding | সহজ — CPU আগেই জানে কোন অংশ কোথায়। | জটিল — CPU কে আগে বের করতে হয় instruction কোথায় শেষ। |
| Execution Speed | দ্রুত (fast execution) | ধীর (comparatively slow) |
| Pipelining | সহজে করা যায়, কারণ সব instruction সমান আকারের। | কঠিন, কারণ instruction-এর দৈর্ঘ্য আলাদা। |
| Control Unit | Hardwired (সহজ, দ্রুত) | Microprogrammed (জটিল, ধীর) |
| Code Size | বড় (প্রতিটি instruction নির্দিষ্ট 32-bit লাগে) | ছোট (ছোট instruction ব্যবহার করা যায়) |
| Memory Usage | বেশি | কম |
| Flexibility | কম — addressing mode সীমিত। | বেশি — অনেক addressing mode ও complex operation সাপোর্ট করে। |
| Design Simplicity | সহজ | জটিল |
| উদাহরণ | ADD R1, R2, R3 (সবসময় 32-bit) | MOV AX, BX (2 byte), MOV EAX, [1234H] (5 byte) |
| প্রধান সুবিধা | দ্রুত ডিকোড ও পাইপলাইন করা সহজ। | কম মেমরি লাগে, কোড ছোট হয়। |
| প্রধান অসুবিধা | কোড সাইজ বড় হয়। | CPU ডিজাইন ও ডিকোড জটিল। |

✅ সারাংশে মনে রাখো:

🔹 Fixed-format (RISC) → সব instruction একই সাইজের ➜ দ্রুত execution
🔹 Variable-length (CISC) → instruction-এর সাইজ ভিন্ন ➜ বেশি ক্ষমতা কিন্তু ধীর গতি

চাও কি আমি এর একটা চিত্র (diagram) তৈরি করে দিই যেখানে Fixed vs Variable instruction-এর বিট স্ট্রাকচার পাশাপাশি দেখানো থাকবে (visual format-এ)?